Pretraining progress

Production EEG encoder (internal name d10) · status as of Oct 30, 2026

| 29.4M parameters 512-wide, 8-layer transformer that reads up to 64 EEG channels | 2.7k h pretraining data 17 datasets, 5.5k people, no labels used | 131k h training exposure 2,525 unique hours × 52 passes, about 21 hours on 4 GPUs | 45 assessments evaluated 30 of 45 reach AUC 0.70 or higher and 15 reach 0.85 or higher (only 2 of those with 30+ test patients) |
| --- | --- | --- | --- |

Where our hours are

28.8k unique hours of EEG are on hand. The production model has been pretrained on 2.7k of them (9%). The rest is processed or converted and available for the next pretraining run.

How the model is trained and checked

Self-supervised pretraining. The encoder reads EEG and learns to predict the next 160 ms of signal, plus 171 standard signal features. No diagnoses or other labels are used.

Checking it on an assessment. The encoder is frozen: each recording becomes a vector of numbers, and a light classifier (TabPFN or XGBoost) is trained on labeled patients and scored on held-out patients.

Fixed, audited test sets. Test cohorts stay the same across model versions, every split is checked for patient overlap (76 of 77 clean; the one leaky split, TBI, was retired), and every model gets the same protocol.

Against standard EEG features. Each assessment is also scored with 28 standard EEG features. On the 17 assessments compared head to head, deep learning features lead on 5, EEG features lead on 4 and 8 are ties (a lead is 0.03 AUC or more; mean difference +0.01 AUC).

Does more pretraining data help? Checks so far

| What we changed | Result | How to read it |
| --- | --- | --- |
| Larger corpus across model generations: 665 h (v6_r3) to 3,261 h (d10), 4.9× | Mean AUC change −0.03 (95% CI −0.07 to +0.01) over 19 classification assessments; 9 up, 10 down. | No measurable gain. Test sets are small (median 20 patients), so the interval is wide. |
| Controlled sweep on the d10 data: 10%, 25%, 50% and 100% of the pretraining hours (253 to 2,525 h), same 3,000 training steps | Validation loss 0.853, 0.866, 0.841, 0.846. | Flat within about ±0.01. One seed; the passes over the data fall as hours rise; short runs. |
| Add Temple clinical EEG to pretraining: 1.65× and 2.8× the hours (3,000-step runs), 31 assessments | AUC change vs the d10-only run: −0.010 (CI −0.043 to +0.023) and +0.002 (CI −0.037 to +0.043). Two d10-only runs differ by −0.030. Abnormal-EEG and seizure assessments are also flat. | No effect beyond run-to-run noise. These short runs score well below the production model (for example 0.70 vs 0.95 AUC on an Alzheimer's assessment), so they show direction, not a ceiling. |

So far, more pretraining data has not improved downstream accuracy in the short tests we have run. A full-length test is next.

What we are doing next

Validation loss versus samples seen: one model on all the data we have, on the full training schedule, with validation loss recorded throughout; compared with models trained on half and a tenth of the data.

Then pretrain on the full Temple set and re-run the frozen-feature evaluation (XGBoost and TabPFN) on the same 45 assessments.

In parallel: a cross-dataset preprocessing audit (reference, units, normalization).

Hours are unique EEG time from file headers (no augmented copies or overlapping windows). Training exposure = unique training hours × passes over them. AUC and the assessments are defined in the slide “How to read these slides”. Sources: Oct 2026 board figures; Temple screening results, Oct 28 2026.